Practical -1

Type of cables

- Twisted-Pair Cables (Ethernet)

These are the most common cables for Local Area Networks (LANs). They consist of pairs of copper wires twisted together to reduce interference.

- Unshielded Twisted Pair (UTP): The standard cable used in most homes and offices. It is     flexible, inexpensive, and easy to install.

- Shielded Twisted Pair (STP): Features foil or braided shielding around each pair of wires to protect data from electromagnetic interference (EMI). It is ideal for industrial environments.

Twisted-pair cables are categorized by performance, such as:

- Cat5e: Supports speeds up to 1 Gbps (100 MHz).

- Cat6: Supports speeds up to 1 Gbps with higher reliability, and 10 Gbps over shorter distances (250 MHz).

- Cat6a: Supports up to 10 Gbps at distances up to 100 meters (500 MHz).

- Cat8: Designed for data centres, supporting up to 40 Gbps (2000 MHz).

- Fiber Optic Cables:

Fiber optic cables transmit data as pulses of light through strands of glass or plastic. They are immune to electromagnetic interference, can span vast distances, and handle massive bandwidths.

- Single-Mode Fiber (SMF):  Uses a tiny core and a single ray of light; built for very long-distance transmissions (up to kilometres).

- Multi-Mode Fiber (MMF): Uses a larger core to send multiple beams of light simultaneously; typically used for shorter distances within buildings or data centres.

- Coaxial Cables : Historically used for cable television (CATV) and early Ethernet networks. Coaxial cables feature a central copper conductor surrounded by insulation and a braided metal shield to block interference. While mostly replaced by Twisted-Pair and Fiber Optic cables in modern LANs, they are still widely utilized for internet delivery to your home (e.g., DOCSIS broadband).

COMMUNICATION MODE:

1. Simplex Mode:

Data travels in one direction only. One device only transmits, while the other only receives.

- Analogy: A one-way street.

- Example: A keyboard sending keystrokes to a computer, or a traditional television broadcast.

2. Half-Duplex Mode:

Data travels in both directions, but not at the same time. While one device transmits, the other          must wait until the channel is clear.

- Analogy: A single-lane bridge where traffic can only cross from one side at a time.

- Example: Walkie-talkies, legacy hub-based Ethernet networks, and Wi-Fi networks.

3. Full-Duplex Mode:

Data travels in both directions simultaneously. Both devices can transmit and receive data at the   exact same moment without interference.

- Analogy: A two-lane highway where cars pass each other in opposite directions.

- Example: Modern switch-based Ethernet networks, fibre optic connections, and mobile phone calls.